CpE 487 Digital Design Lab

Lab 4: Hex Calculator

- Introduction

In this lab, we will program the FPGA on the Nexys2 board to function as a simple hexadecimal calculator capable of adding and subtracting four-digit hexadecimal numbers. We will use a 16-buttom keypad to input hex numbers and the push-buttons on the Nexys2 board to act as operation buttons (+, –, = and clear). The results will be displayed on the four 7-segment displays on the Nexys2 board. The reason we build a hex calculator (rather than a decimal one) is that it makes the 7-segment display operation much simpler. Converting the output to decimal format would require a large amount of additional VHDL code.

- Keypad Interface

We will be using a 16-button keypad module PmodKYPD which plugs into one of the PMOD connectors on the Nexys2 board. The module and its connections are shown in Figure 1. The 16-buttons are labeled “0 1 2 3 4 5 6 7 8 9 A B C D E F”, so we can use this keyboard to enter numbers in hexadecimal format. Documentation on the module can be found on the course website.

Figure 1 PmodKYPD 16-button Keypad

The keyboard  is set up as a matrix in which each row of buttons from left to right are tied to a row pin, and each column from top to bottom is tied to a column pin. There are four row pins and four column pins. When a button is pushed, it connects its row pin to its column pin.

Each row is normally held high (‘1’) by a pull-up resistor connected to VCC. To test the buttons in a particular column, that column should be driven low (‘0’), while all other columns are held high. If a button in that row is depressed while its column pin is low, it will pull the corresponding row pin low. The FPGA can therefore read the state of the buttons by driving the column lines and reading the row lines. The FPGA “walks” a logic ‘0’ through each column line (in turn, while keeping the other column lines high) and reads the row pins to determine if any buttons have been pushed.

- Hardware Setup

Plug the PmodKYPD module into PMOD jack JA1 on the Nexys2 boards as shown in Figure 2. Note that the PmodKYPD uses all 12 pins of jack JA1 and must be connected with a 12 pin cable.  Note the white alignment marks on the connectors as shown in Figure 2. The alignment mark should be on the top-right for the connector attaching to the Nexys2 board and the top-left for the connector attaching to the PmodKYPD module. Please be careful not to bend the pins on the PmodKYPD module.

Figure 2 PmodKYPD Module connected to PMOD jack JA1

- Configuring the FPGA

4.1 Create a New Project

Use the Xilinx ISE software to create a new project named Hexcalc using the same project settings as in Labs 1 and 2.

4.2 Add Source for  “keypad”

Create a new VHDL source module called keypad and load the following source code into the edit window.  Expand the Synthesize command in the Process window and run Check Syntax to verify that you have entered the code correctly.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity keypad is

Port ( samp_ck : in  STD_LOGIC;				-- clock to strobe columns

col : out  STD_LOGIC_VECTOR (4 downto 1);		-- output column lines

row : in  STD_LOGIC_VECTOR (4 downto 1);		-- input row lines

value : out  STD_LOGIC_VECTOR (3 downto 0);	-- hex value of key depressed

hit : out  STD_LOGIC);					-- indicates when a key has been pressed

end keypad;

architecture Behavioral of keypad is

signal CV1, CV2, CV3, CV4: std_logic_vector (4 downto 1) := "1111";  -- column vector of each row

signal curr_col: std_logic_vector (4 downto 1) := "1110";		-- current column code

begin

-- This process synchronously tests the state of the keypad buttons. On each edge of samp_ck,

-- this module outputs a column code to the keypad in which one column line is held low while the

-- other three column lines are held high. The row outputs of that column are then read

-- into the corresponding column vector. The current column is then updated ready for the next

-- clock edge. Remember that curr_col is not updated until the process suspends.

strobe_proc: process

begin

wait until rising_edge(samp_ck);

case curr_col is

when "1110" =>

CV1 <= row; curr_col <= "1101";

when "1101" =>

CV2 <= row; curr_col <= "1011";

when "1011" =>

CV3 <= row; curr_col <= "0111";

when "0111" =>

CV4 <= row; curr_col <= "1110";

when others =>

curr_col <= "1110";

end case;

end process;

-- This process runs whenever any of the column vectors change. Each vector is tested to see

-- if there are any '0's in the vector. This would indicate that a button had been pushed in

-- that column. If so, the value of the button is output and the hit signal is assereted. If

-- not button is pushed, the hit signal is cleared

out_proc: process (CV1, CV2, CV3, CV4)

begin

hit <= '1';

if 	CV1(1) = '0' then value <= X"1";

elsif CV1(2) = '0' then value <= X"4";

elsif CV1(3) = '0' then value <= X"7";

elsif CV1(4) = '0' then value <= X"0";

elsif CV2(1) = '0' then value <= X"2";

elsif CV2(2) = '0' then value <= X"5";

elsif CV2(3) = '0' then value <= X"8";

elsif CV2(4) = '0' then value <= X"F";

elsif CV3(1) = '0' then value <= X"3";

elsif CV3(2) = '0' then value <= X"6";

elsif CV3(3) = '0' then value <= X"9";

elsif CV3(4) = '0' then value <= X"E";

elsif CV4(1) = '0' then value <= X"A";

elsif CV4(2) = '0' then value <= X"B";

elsif CV4(3) = '0' then value <= X"C";

elsif CV4(4) = '0' then value <= X"D";

else hit <= '0'; value <= X"0";

end if;

end process;

col <= curr_col;

end Behavioral; _______________________________________________________________________________

This module synchronously tests the state of the keypad buttons. On each rising edge of the clock samp_ck (approx. every 1.5 ms) the module outputs a code to the 4 column lines of the switch matrix, a code which will enable exactly one column. The first time it outputs 1110, setting column 1 low and all others high. On the second edge, it outputs 1101, setting column 2 low and all others high, continuing to columns 3 and 4 and then returning to column 1 to repeat the process. Each time it enables one of the columns, it reads the row lines into a column vector to determine if any button in that column has been pushed. A separate process runs whenever any of the column vectors change. When that happens, the process searches for a ‘0’ in any of the column vectors. If it finds a ‘0’, it outputs the appropriate hex value and asserts the hit signal. If there are no ‘0’s in the column vectors (meaning no buttons are currently being pushed), it clears the hit signal. The code assumes that only one button is pushed at a time.

4.2 Add Source for  “leddec16”

Create a new VHDL source module called leddec16 and load the following source code into the edit window.  Expand the Synthesize command in the Process window and run Check Syntax to verify that you have entered the code correctly.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity leddec16 is

Port (	dig : in  STD_LOGIC_VECTOR (1 downto 0);		-- which digit to currently display

data : in STD_LOGIC_VECTOR (15 downto 0);		-- 16-bit (4-digit) data

anode: out STD_LOGIC_VECTOR (3 downto 0);	-- which anode to turn on

seg : out  STD_LOGIC_VECTOR (6 downto 0));	-- segment code for current digit

end leddec16;

architecture Behavioral of leddec16 is

signal data4: STD_LOGIC_VECTOR (3 downto 0);	-- binary value of current digit

begin

-- Select digit data to be displayed in this mpx period

data4 <= data(3 downto 0) when dig="00" else		--digit 0

data(7 downto 4) when dig="01" else		--digit 1

data(11 downto 8) when dig="10" else		--digit 2

data(15 downto 12);				--digit 3

-- Turn on segments corresponding to 4-bit data word

seg <=	"0000001" when data4="0000" else	--0

"1001111" when data4="0001" else	--1

"0010010" when data4="0010" else	--2

"0000110" when data4="0011" else	--3

"1001100" when data4="0100" else	--4

"0100100" when data4="0101" else	--5

"0100000" when data4="0110" else	--6

"0001111" when data4="0111" else	--7

"0000000" when data4="1000" else	--8

"0000100" when data4="1001" else	--9

"0001000" when data4="1010" else	--A

"1100000" when data4="1011" else	--B

"0110001" when data4="1100" else	--C

"1000010" when data4="1101" else	--D

"0110000" when data4="1110" else	--E

"0111000" when data4="1111" else	--F

"1111111";

-- Turn on anode of 7-segment display addressed by 2-bit digit selector dig

anode <=	"1110" when dig="00" else	-- digit 0

"1101" when dig="01" else	-- digit 1

"1011" when dig="10" else	-- digit 2

"0111" when dig="11" else	-- digit 3

"1111";

end Behavioral; ______________________________________________________________________________

This module is similar to the code developed in Lab 2 to display a 16-bit hex counter. The four 7-segment displays are time multiplexed using an input signal dig which counts from 0 to 3 at a rate of approx. 380 Hz. The four digits are thus refreshed approx. 95 times per second.  The segment code determines which anode to take low (‘0’). It also determines which 4-bits of the 16-bit data to display. These four bits are translated into a 7-segment code and output to the common display segment lines.

4.3 Add Source for top level  “hexcalc”

Create a new VHDL source module called hexcalc and load the following source code into the edit window.  Expand the Synthesize command in the Process window and run Check Syntax to verify that you have entered the code correctly.

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

use IEEE.STD_LOGIC_UNSIGNED.ALL;

entity hexcalc is

Port ( 	clk_50MHz : in  STD_LOGIC;				  -- system clock (50 MHz)

SEG7_anode : out STD_LOGIC_VECTOR (3 downto 0);  -- anodes of four 7-seg displays

SEG7_seg : out STD_LOGIC_VECTOR (6 downto 0);	  -- common segments of 7-seg displays

bt_clr: in STD_LOGIC;			-- calculator "clear" button

bt_plus: in STD_LOGIC;			-- calculator "+" button

bt_eq: in STD_LOGIC;				-- calculator "-" button

KB_col: out STD_LOGIC_VECTOR (4 downto 1);	-- keypad column pins

KB_row: in STD_LOGIC_VECTOR (4 downto 1 ));	-- keypad row pins

end hexcalc;

architecture Behavioral of hexcalc is

component keypad is

Port ( samp_ck : in  STD_LOGIC;

col : out  STD_LOGIC_VECTOR (4 downto 1);

row : in  STD_LOGIC_VECTOR (4 downto 1);

value : out  STD_LOGIC_VECTOR (3 downto 0);

hit : out  STD_LOGIC);

end component;

component leddec16 is

Port ( dig : in  STD_LOGIC_VECTOR (1 downto 0);

data : in STD_LOGIC_VECTOR (15 downto 0);

anode: out STD_LOGIC_VECTOR (3 downto 0);

seg : out  STD_LOGIC_VECTOR (6 downto 0));

end component;

signal cnt: std_logic_vector(20 downto 0);  	-- counter to generate timing signals

signal kp_clk, kp_hit, sm_clk: std_logic;

signal kp_value: std_logic_vector (3 downto 0);

signal nx_acc, acc: std_logic_vector (15 downto 0);	 -- accumulated sum

signal nx_operand, operand: std_logic_vector (15 downto 0);	-- operand

signal display: std_logic_vector (15 downto 0);			-- value to be displayed

signal led_mpx: STD_LOGIC_VECTOR (1 downto 0);		-- 7-seg multiplexing clock

type state is (ENTER_ACC, ACC_RELEASE, START_OP, OP_RELEASE,

ENTER_OP, SHOW_RESULT);	-- state machine states

signal pr_state, nx_state: state;			-- present and next states

begin

ck_proc: process(clk_50MHz)

begin

if rising_edge( clk_50MHz) then 	-- on rising edge of clock

cnt <= cnt+1;			-- increment counter

end if;

end process;

kp_clk <= cnt(15);				-- keypad interrogation clock

sm_clk <= cnt(20);				-- state machine clock

led_mpx <= cnt(18 downto 17);			-- 7-seg multiplexing clock

kp1:	keypad port map (samp_ck => kp_clk, col => KB_col,

row => KB_row, value => kp_value, hit => kp_hit);

led1:	leddec16 port map (dig => led_mpx, data => display,

anode => SEG7_anode, seg => SEG7_seg);

sm_ck_pr: process (bt_clr, sm_clk)	-- state machine clock process

begin

if bt_clr = '1' then				-- reset to known state

acc <= X"0000";

operand <= X"0000";

pr_state <= ENTER_ACC;

elsif rising_edge (sm_clk) then 			-- on rising clock edge

pr_state <= nx_state;			-- update present state

acc <= nx_acc;				-- update accumulator

operand <= nx_operand;		-- update operand

end if;

end process;

-- state maching combinatorial process

-- determines output of state machine and next state

sm_comb_pr: process (kp_hit, kp_value, bt_plus, bt_eq, acc, operand, pr_state)

begin

nx_acc <= acc;			-- default values of nx_acc, nx_operand & display

nx_operand <= operand;

display <= acc;

case pr_state is			-- depending on present state...

when ENTER_ACC =>		-- waiting for next digit in 1st operand entry

if kp_hit = '1' then

nx_acc <= acc(11 downto 0) & kp_value;

nx_state <= ACC_RELEASE;

elsif bt_plus = '1' then

nx_state <= START_OP;

else nx_state <= ENTER_ACC;

end if;

when ACC_RELEASE =>		-- waiting for button to be released

if kp_hit = '0' then

nx_state <= ENTER_ACC;

else nx_state <= ACC_RELEASE;

end if;

when START_OP =>		-- ready to start entering 2nd operand

if kp_hit = '1' then

nx_operand <= X"000" & kp_value;

nx_state <= OP_RELEASE;

display <= operand;

else nx_state <= START_OP;

end if;

when OP_RELEASE =>		-- waiting for button ot be released

display <= operand;

if kp_hit = '0' then

nx_state <= ENTER_OP;

else nx_state <= OP_RELEASE;

end if;

when ENTER_OP =>		-- waiting for next digit in 2nd operand

display <= operand;

if bt_eq = '1' then

nx_acc <= acc + operand;

nx_state <=SHOW_RESULT;

elsif kp_hit = '1' then

nx_operand <= operand(11 downto 0) & kp_value;

nx_state <= OP_RELEASE;

else nx_state <= ENTER_OP;

end if;

when SHOW_RESULT =>		-- display result of addition

if kp_hit = '1' then

nx_acc <= X"000" & kp_value;

nx_state <= ACC_RELEASE;

else nx_state <= SHOW_RESULT;

end if;

end case;

end process;

end Behavioral;

_____________________________________________________________________

This is the top level that hooks it all together. This module:

- Creates an instance of the keypad interface and 7-segment decoder interface modules

- Make connection to the external keypad, display and buttons

- Has a timing process to generate clocks for the keypad, display multiplexer and state machine

- Implements a finite state machine to implement the operations of the calculator in response to button pushes.

The finite state machine uses a number of variables to keep track of the addition operation. The variable acc is an accumulator that holds the current summation result. The variable operand holds the value of the next operand that will be added to the accumulator. The variable display holds the value currently being displayed on the 7-segment displays. The variable pr_state is the current state of the state machine. Depending on the current state, the machine will react to pushed  keypad buttons or operation buttons to update variables, change the output and select the next state.  Operation of the state machine is summarized in Figure 3.

Figure 3 Calculator State Transition Diagram

When the clear button is pushed, the machine enters the ENTER_ACC state. In this state the machine waits for a keypad button to be pushed. When a keypad button is pushed,  the code adds the new digit to the 16-bit word in the accumulator and then waits in the ACC_RELEASE state for the button to be released. It then returns to the ENTER_ACC state to wait for the next digit. This process continues until the “+” button is pushed. The machine then enters the START_OP state where it waits for the first digit of the second operand. Once a keypad button has been pushed, it records the hex digit and then enters the OP_RELEASE state waiting for the keypad button to be released. Once released, the machine enters the ENTER_OP state where it continues to received operand digits each time a keypad button is pushed. This continues until the user presses the “=’ button at which point it performs the addition and goes to the SHOW_RESULT state. Once in the SHOW_RESULT state, it shows the result of the addition and waits for a keypad button push to start a new calculation.

4.4 Synthesis and Implementation

Highlight the hexcalc module in the Hierarchy window and execute the Synthesis command in the Process window.

Add an Implementation Constraint source file hexcalc.ucf and enter the following data into the edit window:

____________________________________________________________________

NET	"clk_50MHz"	 LOC = B8;

NET	"KB_col(1)"	 LOC = M15;

NET	"KB_col(2)"	 LOC = L17;

NET	"KB_col(3)"	 LOC = K12;

NET	"KB_col(4)"	 LOC = L15;

NET	"KB_row(1)"	 LOC = M16;

NET	"KB_row(2)"	 LOC = M14;

NET	"KB_row(3)"	 LOC = L16;

NET	"KB_row(4)"	 LOC = K13;

NET	"bt_clr"		LOC = H13;	# BTN3

NET	"bt_plus"	LOC = B18;	# BTN0

NET	"bt_eq"		LOC = D18;	# BTN1

NET "SEG7_seg[0]" LOC = H14;

NET "SEG7_seg[1]" LOC = J17;

NET "SEG7_seg[2]" LOC = G14;

NET "SEG7_seg[3]" LOC = D16;

NET "SEG7_seg[4]" LOC = D17;

NET "SEG7_seg[5]" LOC = F18;

NET "SEG7_seg[6]" LOC = L18;

NET "SEG7_anode[0]" LOC = F17;

NET "SEG7_anode[1]" LOC = H17;

NET "SEG7_anode[2]" LOC = C18;

NET "SEG7_anode[3]" LOC = F15; __________________________________________________________________

The configuration file sets up push-button BTN0 as the “+” key, BTN1 as the “=” key and BTN3 as the “clear” key.

Now highlight the hexcalc module in the Hierarchy window and run Implement Design followed by Generate Programming File (don’t forget to change the FPGA Start-up Clock to be the JTAG Clock).

4.5  Download and Run

Use the Adept software to download your configuration file hexcalc.bit. Once loaded, you should see “0000” appear on the seven segment displays. Enter a multi-digit hex number using the keypad. It should appear, once character at a time on the 7-segments displays. Once you have entered the first operand, press the “+” key (BTN0). Now enter the second operand and press the “=” key (BTN1). The value of the sum of the operands should now appear on the display. You can now start a second calculation by once again entering the first operand using the keypad. The “clear” key (BTN3) should always set the result on the display to zero.

4.6  Now let’s make some changes …

Modify your VHDL code to do the following:

- Modify the leddec16 module to that it performs leading zero suppression (i.e. it does not display zeros ahead of the first non-zero digit). For example, the number “0023” should appear as “23” with the leading zeros suppressed. Hint: You can turn off any digit in the display by never taking its anode to ‘0’. Modify the “anode <=” conditional assignment statement in leddec16 so that it only turns on a particular digit if it is non-zero or if there is non-zero information in the higher order digits of the data word.

- Expand the calculator to also do subtraction operations. Use the button BTN2 (pin E18 on the Nexys2 board) as the “–” key. Hint: Modify the ENTER_ACC state to also test for the “–” key being depressed. Create a new signal which will record whether the “+” key or the “–” key was pushed. Then, when in state ENTER_OP and the “=” key is pressed, test your new signal to determine whether you should do an addition or a subtraction.